Experiment No. 9 (Linear Block Code)

Code :

clc;

clear all;

close all;

n = input('Enter the length of codeword(n):');

k = input('Enter the length of message bits(k):');

parity_matrix = input('Enter the parity matrix(size k x (n-k)):');

identity_matrix = eye(k);

generator_matrix=[identity_matrix, parity_matrix];

disp('The generator matrix (G) is:');

disp(generator_matrix);

messages=0:(2^k)-1;

binary_messages = de2bi(messages,k,'left-msb');

disp('Messages are:');

disp(binary_messages);

codewords = mod(binary_messages*generator_matrix,2);

disp('Codewords are:');

disp(codewords);

received_codeword = input('Enter received codeword:');

parity_transpose = parity_matrix';

identity_matrix_extended = eye(n-k);

parity_check_matrix = [parity_transpose, identity_matrix_extended];

parity_check_matrix_transpose = parity_check_matrix';

disp('The H transpose matrix is:');

disp(parity_check_matrix_transpose);

syndrome = mod(received_codeword*parity_check_matrix_transpose,2);

disp('The syndrome of the received codeword is:');

disp(syndrome);

if all(syndrome == 0)

disp('No error is present.');

else

disp('Error is present, need for error correction.');

error_patterns = eye(n);

disp('Error pattern matrix is:');

disp(error_patterns);

syndrome_patterns = mod(error_patterns*parity_check_matrix_transpose,2);

disp('Syndrome patterns are:');

disp(syndrome_patterns);

[~, error_index] = ismember(syndrome, syndrome_patterns, 'rows');

if (error_index > 0)

corrected_codeword = xor(received_codeword, error_patterns(error_index, :)); % fixed syntax here

disp('Corrected codeword is:');

disp(corrected_codeword);

corrected_message = corrected_codeword(1:k);

disp('The corrected message is:');

disp(corrected_message);

else

disp('Error pattern not found. Unable to correct.');

end

end

Output :

Enter the length of codeword(n):6

Enter the length of message bits(k):3

Enter the parity matrix(size k x (n-k)):[1 1 1; 1 1 0; 0 1 1]

The generator matrix (G) is:

1     0     0     1     1     1

0     1     0     1     1     0

0     0     1     0     1     1

Messages are:

0     0     0

0     0     1

0     1     0

0     1     1

1     0     0

1     0     1

1     1     0

1     1     1

Codewords are:

0     0     0     0     0     0

0     0     1     0     1     1

0     1     0     1     1     0

0     1     1     1     0     1

1     0     0     1     1     1

1     0     1     1     0     0

1     1     0     0     0     1

1     1     1     0     1     0

Enter received codeword:[1 0 1 1 0 1]

The H transpose matrix is:

1     1     1

1     1     0

0     1     1

1     0     0

0     1     0

0     0     1

The syndrome of the received codeword is:

0     0     1

Error is present, need for error correction.

Error pattern matrix is:

1     0     0     0     0     0

0     1     0     0     0     0

0     0     1     0     0     0

0     0     0     1     0     0

0     0     0     0     1     0

0     0     0     0     0     1

Syndrome patterns are:

1     1     1

1     1     0

0     1     1

1     0     0

0     1     0

0     0     1

Corrected codeword is:

1   0   1   1   0   0

The corrected message is:

1   0   1